HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone: +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                                 16/04/2014

## SECTION VIII

## ABSTRACT PRICE SCHEDULE

Sub: Electrical work of Hospital Projects works at Himachal Pradesh  :   Mandi  District,:-

| Sl. No. | Details of Designated Site (Hospital Location) | Project reference |  |
| --- | --- | --- | --- |
| 1 | Public Health Centre, Nangwain, Mandi, Himachal Pradesh | Annexure 1 | a) ELECTRICAL WORK OF NBCC |
| 2 | Community Health Centre, Rohanda, Mandi, Himachal Pradesh | Annexure 2 | a) ELECTRICAL WORK OF NBCC |
| 3 | Community Health Centre, Dehar, Mandi, Himachal Pradesh | Annexure 3 | a) ELECTRICAL WORK OF NBCC |
| 4 | Community Health Centre, Kottli, Mandi, Himachal Pradesh | Annexure 4 | a) ELECTRICAL WORK OF NBCC |
| 5 | Civil Hospital, Karsog Mandi, Himachal Pradesh | Annexure 5 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBCC at OT |
|  |  |  | c) ELECTRICAL WORK OF NBSU |
|  |  |  | d) ELECTRICAL WORK OF BSC |

DRAWING ATTACHED

### The Tenderer has to submit their quote for all the schedules of the respective Tender.  If a tenderer is not quoting for all the schedules of the respective Tender, the tender will be  summarily rejected. HLL will place Work Order to the tenderer, whose total rate for all the hospital of the respective tender comes to lowest

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                      16/04/2014

## HLL LIFECARE LIMITED

Akkulam Factory

THIRUVANANTHAPURAM-695017

HIMACHAL PRADESH  :  MANDI DISTRICT

ELECTRICAL WORK

# ABSTRACT PRICE BID

| Sl. No. | Details of Designated Site (Hospital Location) |  | Amount (Rs) |
| --- | --- | --- | --- |
| 1 | Public Health Centre, Nangwain, Mandi, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
| 2 | Community Health Centre, Rohanda, Mandi, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
| 3 | Community Health Centre, Dehar, Mandi, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
| 4 | Community Health Centre, Kottli, Mandi, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
| 5 | Civil Hospital, Karsog Mandi, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBCC at OT |  |
|  |  | c) ELECTRICAL WORK OF NBSU |  |
|  |  | d) ELECTRICAL WORK OF BSC |  |
| TOTAL Rs: |  |  |  |

The Tenderer has to submit their quote for all the schedules of the respective Tender.  If a tenderer is not quoting for all the schedules of the respective Tender, the tender will be  summarily rejected. HLL will place Work Order to the tenderer, whose total rate for all the hospital of the respective tender comes to lowest

SEAL OF THE APPLICANT

Date:                                                                           SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone: +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                                          16/04/2014

## PRICE SCHEDULE

## (Public Health Centre, Nangwain)

[Annexure 1 (a)]

| NBCC : ELECTRICAL WORKS FOR Public Health Centre, Nangwain |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 1 |  |  |
|  | 6mod | Each | 1 |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL Rs: |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Rohanda)

[Annexure 2 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Rohanda |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Dehar)

[Annexure 3 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Dehar |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Kottli)

[Annexure 4 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Kottli |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required..(Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Karsog)

[Annexure 5 (a)]

| NBCC : ELECTRICAL WORKS FOR Civil Hospital, Karsog |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Karsog)

[Annexure 5 (b)]

| ELECTRICAL WORKS FOR NBCC-OT Civil Hospital, Karsog |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ESTIMATE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N) | Each | 1.00 |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required..(Make: Legrand/HPL) | Each | 2.00 |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 1 |  |  |
|  | 6mod | Each | 1 |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand/Crabtree) |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Karsog)

[Annexure 5 (c)]

| ELECTRICAL WORKS FOR NBSU AT CH KARSOG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 7 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 75 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 175 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 100 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 22 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 22 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 22 |  |  |
|  | 15/16 Amps switch | Each | 10 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 10 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20 mm | Metre | 40 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 6 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 30 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 4 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, legrand) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL Rs: |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-010	                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Karsog)

[Annexure 5 (d)]

| ELECTRICAL WORKS FOR BSC AT CH KARSOG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for Havells point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 3 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 20 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 75 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 130 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 8 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 7 |  |  |
| b | 6 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 4 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
| c | 15/16 Amps switch | Each | 6 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 6 |  |  |
| 7 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 30 |  |  |
| 8 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 2 |  |  |
| 9 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
| a | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 45 |  |  |
| b | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 75 |  |  |
| c | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 25 |  |  |
| 10 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
| a | 2 X 6 sq. mm | Each | 6 |  |  |
| 11 | Supply and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 2 pole and earth, metal enclosed plug top along with 20 amps "C" curve, SP, MCB in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket outlet and complete with connections, testing and commissioning etc as required. | Each | 2 |  |  |
| 12 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 2 |  |  |
| 13 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 13.1 | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 13.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for Havellsing and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 14 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 16/25A COS | Each | 1 |  |  |
| 15 | WORK COMPLETION |  |  |  |  |
| 15.1 | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 15.2 | Obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. | LS | 1 |  |  |
|  | Miscellaneous Items |  |  |  |  |
| 16 | Miscellaneous electrical work, removing of existing conduits and interconnections of Havells,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL Rs: |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT