Amendment / Corrigendum to TENDER

## Sub:TENDER FOR SUPPLY, INSTALLATION & COMMISSIONING OF :

## OTO ACOUSTIC EMISSION MACHINE TO SNCUs AT  HIMACHAL PRADESH.

Ref: IFB NO: HLL/AFT-CMO/HP/SNCU-EQP(OAE)/2017-18 DATED 26.12.2017

| Sl No | Original Tender Details | # Amended Tender Details |
| --- | --- | --- |
| 1. | Bid Submission Closing Date 10/01/2018 at 14.00 hrs Bid Opening Date 10/01/2018 at 15.00 hrs | Bid Submission Closing Date 17/01/2018 at 14.00 hrs Bid Opening Date 17/01/2018 at 15.00 hrs |
| CORRIGENDUM / AMENDMENT TO SECTION VI-Technical Specification (Page – 39) |  |  |
| REVISED SPECIFICATIONS FOR OAE SYSTEM FOR NEWBORN SCREENING - Portable standalone unit weighing less than 500g. - Unit should have facility to measure DPOAE and TEOAE both. - The results should be displayed on LCD touch screen along with graph and response spectrum. - Unit should run on inbuilt re-chargeable batteries. - Unit should be CE approved with safety of BF type - Should be possible to enter a check-up type, enter a patient’s data, and keep a record of checkups database. - Unit should have facility of storing at least 10000 tests in memory. - DPOAE should test from 500 to 12000 Hz, with at least 2 points per octave. - Equipment should have option put memory card of upto 64 GB - TEOAE should have analysis epoch of 20 ms& frequency bands for analysis should be from 1 to 5 kHz. - Stimulation level range to be 50 to 80 dB SPL. - Should be possible to process and archive the check up results on the external computer. - Should have inbuilt wireless Bluetooth connectivity to enable the unit to connect wirelessly to printer and external computer. - Accessories to include set of ear tips for newborn ear tips, OAE probe, Set of OAE probe tips, test cavity and power supply unit. - It should have Bluetooth printer - Equipment should have facility to customize the protocol - Equipment should have facility to change the pass/ refer criteria |  |  |

All other terms and conditions in the original tender remain unchanged.

10.01.2018                                                                        Joint General Manager (Materials)