HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

## TENDER FOR RENOVATION OF DIALYSIS ROOM ON TURNKEY BASIS

## (CIVIL AND ELECTRICAL  WORKS)

## FOR

PRIMARY HEALTH CENTRE, EDAKKADU, KANNUR DISTRICT

KERALA

### IFB NO. HLL/AFT-CMO/KL-DU/CIV&ELE/2017-18/01, DT. 01.04.2017

THIRUVANANTHAPURAM – 17

Phone +91 471 244 5930, Fax      +91 471 244 5935

Email: hcdcmo@lifecarehll.com

Web: www.lifecarehll.com

##### HLL LIFECARE LIMITED

AKKULAM  FACTORY,   SREEKARIAM  P.O.  THIRUVANANTHAPURAM - 17

TENDER FOR RENOVATION OF DIALYSIS ROOM AT PRIMARY HEALTH CENTRE, EDKKADU, KANNUR DISTRICT, KERALA ON TURNKEY BASIS

## (CIVIL AND ELECTRICAL  WORKS)

BID REFERENCE	                 		:         	HLL/AFT-CMO/KL-DU/CIV&ELE/2017-18/01,                    							DT. 01.04.2017

DATE OF COMMENCEMENT

OF SALE OF BIDDING DOCUMENT	             :	01.04.2017

LAST DATE FOR SALE OF BIDDING

DOCUMENT					: 	07.04.2017, 15.00 Hrs.

LAST DATE AND TIME FOR

RECEIPT OF BIDS				: 	10.04.2017,   10.00 Hrs.

TIME AND DATE OF OPENING

OF BIDS					: 	10.04.2017,  11.00 Hrs.

PLACE OF OPENING OF BIDS		:	HLL LIFECARE LIMITED

AKKULAM  FACTORY,

SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone +91 471 244 5930

Fax      +91 471 244 5935

Email: hcdcmo@lifecarehll.com

ADDRESS FOR COMMUNICATION		:	Joint General Manager (Mtls)

HLL LIFECARE LIMITED

AKKULAM  FACTORY,

SREEKARIAM  P.O.

THIRUVANANTHAPURAM - 17							                                          Phone +91 471 244 5930

Fax      +91 471 244 5935

Email: hcdcmo@lifecarehll.com

ABSTRACT

SECTION No.		DESCRIPTION	                                  PAGE No.

PART  A

SECTION I		INVITATION FOR BIDS (IFB)				     04

SECTION II		INSTRUCTION TO BIDDERS (ITB)			     05

SECTION III		GENERAL CONDITIONS OF CONTRACT (GCC)	     14

SECTION IV		SPECIAL CONDITIONS OF CONTRACT (SCC)	     20

SECTION V		QUALIFICATION CRITERIA				     22

SECTION VI		PROFORMA ABOUT THE CONTRACTOR		    23

SECTION VII		PERFORMANCE STATEMENT			     24

SECTION VIII              INDEMNITY CLAUSE                                                          25

SECTION IX                 DECLARATION                                                                     26

PART : B

SECTION X		ABSTRACT PRICE SCHEDULE			    28

SECTION XI		PRICE SCHEDULE (I)					    29

## PART – A

SECTION I

INVITATION FOR BIDS (IFB)

Sealed and super scribed tenders, under Two Bid system, is invited from the eligible contractors, for execution of Renovation of Dialysis Room at Primary Health Centre,  Edakkadu, Kannur District on turnkey basis. The detailed work is given as in the Price Schedule to this Tender.

| Sl. No. | Details of Sites in the Hospitals | EMD Amount (Rs) |
| --- | --- | --- |
| 1 | Dialysis Room at Primary Health Centre, Edakkadu, Kannur District, Kerala on turnkey basis | 36,000.00 |

1. Interested eligible Bidders may obtain further information from the Office of the                              Joint General Manager (Mtls), HLL Lifecare Ltd, Akkulam Factory, Thiruvananthapuram -17, Kerala

2.   Deleted.

3.   The Tender Document can be downloaded from our Website www.lifecarehll.com.

4. Interested eligible bidders may submit their bidding documents at the Office of the                         Joint General Manager (Mtls), HLL Lifecare Ltd, Akkulam Factory, Thiruvananthapuram –17 on the specified date and time.

5.  All bids must be accompanied by the documents specified herein and must be delivered to Joint General Manager (Mtls) on or before 10.04.2017, 10.00 Hrs.  The Technical Bids will be opened on 10.04.2017, at 11.00 Hrs, in the presence of the bidders or their duly authorized representatives who wish to attend the bid opening on the specified date and time at HLL's office mentioned above. In the event of the date being declared is a closed holiday for HLL, the due date for submission of bids and opening of bids will be the following working day at the appointed time.

SECTION II

INSTRUCTION TO BIDDERS

A.INTRODUCTION

1. Eligible Bidders

1.1	Only experienced contractors are eligible to participate in the tender.

The tenderer shall furnish, as part of its tender, relevant details and documents establishing its eligibility to quote and its qualifications to perform the contract if its tender is accepted.

1.2  The documentary evidence needed to establish the tenderer’s qualifications shall fulfill the following requirements:

a) The tenderer has the required financial, technical and service capability necessary to perform the contract and, further, it meets the qualification criteria incorporated in the Section V in these documents.

1.3 a) The bidder should have Minimum 3 years experience in the relevant field (SECTION V)

b) The Tenderer should have executed satisfactorily at least 3 jobs preferably of similar kind (CIVIL AND ELECTRICAL  WORKS) as mentioned in SECTION V – Qualification Criteria

1.4 	Bidder submit annual turnover statement for last 3 financial years, each year not less than atleast 25 lakhs, certified by Chartered Accountant.

1.5   The bidders should quote for all the schedules in the Tender, failing which, such bids shall be considered as incomplete and will be summarily rejected.

2. Cost of Bidding

2.1 	Cost of bidding shall be completely on the tenderer.

B. THE BIDDING DOCUMENTS

3. Contents of Bidding Documents

3.1 	The work required, bidding procedures and contract terms are prescribed in the Bidding documents. In addition to the Invitation for Bids, the Bidding documents include:

a.	Instruction to Bidders (ITB);

b.	General Conditions of Contract (GCC);

c.	Special Conditions of Contract (SCC);

d. 	Qualification criteria;

e.	Proforma bout the Contractor

f.       Performance statement

g.       Indemnity Clause

h.       Declaration

i.	Abstract Price Schedule

j.	Price Schedules

3.2 	The Bidder is expected to examine all instructions, forms, terms and specifications in the Bidding Documents. Failure to furnish all information required by the Bidding Documents or submission of a bid not substantially responsive to the Bidding Documents in every respect will be at the Bidder’s risk and may result in rejection of its bid.

4. Clarification of Bidding Documents

4.1  	The Bidder, who require any clarification of the Bidding Documents may notify the Company in writing at the Company’s mailing address indicated in the Invitation for Bids. The Company will respond in writing to any such request which is received not later than 3 days prior to the deadline for submission of bids prescribed by the Company. Any such response (relevant) against the clarification of the tenderer shall be notified in our website only.

5. Amendment of Bidding Documents

5.1  	At any time prior to the deadline for submission of bids, the Company may, for any reason, whether at its own initiative or in response to a clarification requested by a prospective bidder, modify the Bidding Documents by an amendment.

Those bidders who download the tender documents from our website should furnish the Name and address of the vendor, name of contact person, telephone and Email details immediately by Email to hcdcmo@lifecarehll.com. Any changes pertains to this tender shall be communicated only through our website www.lifecarehll.com

5.2 	All prospective bidders who have received the Bidding Documents will be notified of the amendment in writing or by cable and will be binding on them.

5.3 	In order to allow prospective bidders reasonable time in which to take the amendment into account in preparing their bid, the Company may, at its discretion, extend the deadline for the submission of bids

C. PREPARATION OF BIDS

6. Language of Bid

6.1      The Bid prepared by the bidder and all correspondence and documents relating to the bid exchanged by the Bidder and the Company, shall be written in the English language. Supporting documents and printed literature furnished by the Bidder may be written in another language provided they are accompanied by an accurate translation of the relevant passages in the English language in which case, for purposes of interpretation of the Bid, the English translation shall govern.

7. Documents Comprising the Bid

Following documents and forms are to be included in the technical bid and commercial bid respectively.

7.1  TECHNICAL BID

The technical bid shall consist of the following documents.

- Tender Fee : DD for Rs. 525.00 (inclusive of tax) drawn in favour of HLL Lifecare Limited payable at Thiruvananthapuram is to be attached towards tender fee.

- EMD/Bid Security for Rs. 36,000.00 in the form of Demand Draft from schedule Bank, drawn in favor of HLL Lifecare Limited, payable at Thiruvananthapuram. The EMD/Bid Security shall be refunded to the non-responsive bidders within 60 days from the date of opening of Bid. The Tender received without EMD will be rejected.

- SSI/MSME units interested in availing exemption from payment of Tender Fee & EMD should submit a valid copy of their registration certificate issued by the concerned DIC or NSIC /  Udyog Aadhaar. But the Party has to provide Security deposit if Tender is awarded to them.

- Copy of Balance sheet / Bank Statement for the past two financial years.

- Performance statement in the Proforma VII.

- Documentary evidence established in accordance with ITB Clause A1 that the Bidder is eligible to bid and is qualified to perform the contract if its bid is accepted;

7.2  PRICE BID

Duly filled Bid Form i.e. Price bid Abstract and Price Schedule as per Price Bid.

7.3 The Work Order shall be placed to the tenderer, whose total rate comes to lowest and responsive.

8. Bid Currencies

8.1 Prices shall be quoted in Indian Rupees.

9. Documents establishing bidder’s eligibility and qualifications

9.1 Pursuant to ITB Clause 1, the bidder shall furnish, as part of its bid, documents establishing the bidder’s eligibility to bid and its qualifications to perform the Contract.

9.2 The documentary evidence of the Bidder’s qualifications to perform the Contract if its bid is accepted, shall establish to the Company’s satisfactions.

(a) that the bidder has the financial and technical capability necessary to perform the Contract and meets the criteria outlined in the qualification requirements specified in Section V. All bids submitted shall include the following information:

Details of experience and past performance of the bidder on items offered and on those of similar nature and details of current contracts in hand and other commitments as per proforma given in Section VII

10. Scope of Documents:

10.1  For the purpose of the documentary evidence to be furnished pursuant to ITB Clause 9.2 above, the Bidder shall note that the standards for workmanship, material and references to brand names or catalogue numbers, if any, designated by the Company in its Tender schedules are intended to be descriptive only and not restrictive.

11. Period of Validity of Bids

11.1  Price Bid shall be valid for One year from the date of opening of Price Bid

11.2 	In exceptional circumstances, the Company may solicit the bidders consent to an extension of the period of validity. The request and the responses thereto shall be made in writing.

12. Format and Signing of Bid.

12.1 	The bidder shall prepare the bid clearly marking as appropriate

12.2  The bid shall be typed or written in indelible ink and shall be signed by the Bidder or a person or persons duly authorized to bind the Bidder to the Contract. The letter of authorization shall be indicated by written power-of-attorney accompanying the bid. All pages of the bid, except for printed literature, shall be initialed by the person(s) signing the bid.

12.3 Any interlineations, erasures or overwriting shall be valid only if they are initialed by the person or person(s) signing the bid.

D. SUBMISSION OF BIDS

#### 13. Sealing and Marking of bids

Separate bids shall be submitted for Price Bid and Technical Bids.

13.1  The bidders shall seal Technical bid   and Price bid separately, duly written on the envelopes as “TECHNICAL BID – Renovation of Dialysis Room at Primary Health Centre, Edakkadu, Kannur District, Kerala on turnkey basis .” - cover “A” and “PRICE BID- Renovation of Dialysis Room at Primary Health Centre, Edakkadu, Kannur District, Kerala on turnkey basis.” - cover “B”.  The Bidders shall put both the envelopes (cover “A” & cover “B”) in an another envelope.

The outer Envelop shall superscribed with ;

“TENDER FOR RENOVATION OF

DIALYSIS ROOM AT PRIMARY HEALTH CENTRE, EDAKKADU, KANNUR DISTRICT, KERALA  ON TURNKEY BASIS

“HLL/AFT-CMO/KL-DU/CIV&ELE/2017-18/01, DT. 01.04.2017”

“DO NOT OPEN BEFORE 11.00 Hrs. on 10.04.2017”

13.2  The inner and outer envelopes shall be addressed to the following address:

Joint General Manager (Materials)

HLL Lifecare Limited,

Akkulam Factory, Sreekaryam  P.O. ,

Thiruvananthapuram -17

Phone 0471 2445930 / 2442642

Email : hcdcmo@lifecarehll.com

- The inner envelopes shall also indicate the name and address of the bidder to enable the bid to be returned unopened in case it is declared “Technically not qualified”.

- If the inner and outer envelopes are not sealed and marked as required, the Company will assume no responsibility for the bid’s misplacement or premature opening.

- Bids must be received by the Company at the address specified not later than the date and time specified in the Invitation For Bid.  In the event of the specified date for the submission of bids, being declared a holiday for the Company, the bids will be received up to the appointed time on the next working day.

- The Company may, at its discretion, extend this deadline for the submission of bids by amending the bidding documents in which case all rights and obligations of the Company and bidders previously subject to the deadline will thereafter be subject to the deadline as extended.

14. LATE BIDS

14.1 Any bid received by the Company after the deadline for submission of bids prescribed by the Company, will be rejected and returned unopened to the bidder.

15.  MODIFICATION AND WITHDRAWAL OF BIDS

15.1 	The bidder may modify or withdraw its bid after the bid submission, provided that written notice of the modification or withdrawal is received by the Company prior to the deadline prescribed for submission of bids.

15.2  The bidder’s modification or withdrawal notice shall be prepared, sealed, marked and dispatched in accordance with the provisions of ITB Clause 15. A withdrawal notice may also be sent by cable, but followed by a signed confirmation copy, post-marked not later than the deadline for submission of bids.

15.3  No bid may be modified subsequent to the deadline for submission of bids.

15.4 	No bid may be withdrawn in the interval between the deadline for submission of bids and the expiration of the period of bid validity specified by the bidder in the Bid Document.

E. BID OPENING AND EVALUATION

16. Opening of Bids by Company

16.1 The Company will open all technical bids (Cover A), in the presence of bidder's representatives who choose to attend, at 11.00 Hrs on 10.04.2017 at the following Office

JOINT GENERAL MANAGER (MATERIALS)

HLL Lifecare Limited,

Akkulam Factory, Sreekaryam  P.O. ,

Thiruvananthapuram -17

Phone 0471 2445930 / 2442642

Email : hcdcmo@lifecarehll.com

16.2 	In the event of the specified date of bid opening being declared a holiday for the Company, the bids shall be opened at the appointed time and location on the next working day.

16.3  The bidder’s names, modifications, bid withdrawals and the presence or absence of the requisite documents and such other details as the Company, at its discretion, may consider appropriate will be announced at the opening. No bid shall be rejected at bid opening, except for late bids, which shall be returned unopened to the bidder.

16.4    Deleted

16.5   The “Price Bid” (Cover B) will be opened after evaluation of “Technical bids” (Cover A) and the date and time will be intimated to bidders whose bids are responsive to and Qualified in the Technical Bid.

16.5 	The Company will scrutinize the technical bid for compliance to the conditions and documentation requirement as per the bid document. The bidders will be short-listed on the basis of responsiveness of technical bid. The price bid of the bidders who are disqualified at the technical scrutiny and on-site evaluation, as applicable, will be returned un-opened. The short listed bidders will be informed about the time, date and venue of the price bid opening. The successful bidder shall be identified on the basis of lowest evaluated substantially responsive bid.

17.  CLARIFICATION OF BIDS

17.1    During evaluation of bids, the Company may, at its discretion, ask the bidder for a clarification of its bid.  The request for clarification and the response shall be in writing and no change in prices or substance of the bid shall be sought, offered or permitted.

18   PRELIMINARY EXAMINATION

18.1   The Company will examine the bids to determine whether they are complete, whether any   computational errors have been made, whether the documents have been properly signed, and whether the bids are generally in order.

18.2   Arithmetical errors will be rectified on the following basis.  If there is a discrepancy between the unit price and the total price that is obtained by multiplying the unit price and quantity, the unit price shall prevail and the total price shall be corrected. If there is a discrepancy between words and figures, the amount in words will prevail.  If the bidder does not accept the correction of the errors, its bid will be rejected.

18.3 	The Company may waive any minor informality or non-conformity or irregularity in a bid, which does not constitute a material deviation, provided such a waiver does not prejudice or affect the relative ranking of any bidder.

18.4   	Prior to the detailed evaluation, pursuant to Clause ITB 21, the Company will determine the substantial responsiveness of each bid to the bidding documents. For purposes of these clauses a substantially responsive bid is one which conforms to all the terms and conditions of the bidding documents without material deviations. Deviations from or objections or reservations to critical provisions such as those concerning Warranty, Force Majeure, Applicable law  and Taxes and Duties  will be deemed to be material deviation. The Company’s determination of a bid’s responsiveness is to be based on the contents of the bid itself without recourse to extrinsic evidence.

18.5    If a bid is not substantially responsive, it will be rejected by the Company and may not subsequently be made responsive by the bidder by correction of the non-conformity.

19. EVALUATION AND COMPARISON OF BIDS

19.1  The Company will evaluate and compare bids previously determined to be substantially responsive, pursuant to ITB Clause 20.

20. CONTACTING THE COMPANY

20.1    No bidder shall contact the Company on any matter relating to its bid, from the time of the bid opening to the time the contract is awarded. If the bidder wishes to bring additional information to the notice of the Company it should do so in writing.

20.2    Any effort by a bidder to influence the Company in its decisions on bid evaluation, bid comparison, or selection may result in the rejection of the bidders bid.

F. AWARD OF CONTRACT

21. POST QUALIFICATION

21.1 	In the absence of pre-qualification, the Company will determine to its satisfaction whether the bidder who has having submitted the lowest evaluated responsive bid meets the criteria specified in ITB Clause 1 and is qualified to perform the contract satisfactorily

21.2 	The determination will take into account the bidders financial, technical capabilities. It will be based upon an examination of the documentary evidence of the bidder’s qualifications submitted by the bidder, pursuant to ITB Clause 1 as well as such other information as the Company deems necessary and appropriate.

21.3 	An affirmative determination will be a prerequisite for award of the contract to the bidder. A negative determination will result in rejection of the bidders bid, in which event the Company will proceed to the next lowest evaluated bid to make a similar determination of that bidder’s capabilities to perform satisfactorily.

22. AWARD CRITERIA

22.1 	Subject to ITB Clause 23, the Company will award the contract to the successful bidder whose bid has been determined to be substantially responsive and as the lowest evaluated bid, provided further that the bidder is determined to be qualified to perform the contract satisfactorily.

23. COMPANY’S RIGHT TO VARY QUANTITIES AT TIME OF AWARD

  - The Company reserves the right at the time of award of contract to increase or decrease the quantity of materials and services originally specified in the bid document without any change in unit price or other terms and conditions within the bid validity period.

24.  COMPANY’S RIGHT TO ACCEPT ANY BID AND TO REJECT ANY OR ALL BIDS

24.1 	The Company reserves the right to accept or reject any bid, and to annul the bidding process and reject all bids at any time prior to award of Contract, without thereby incurring any liability to the affected bidder or bidders.

25. NOTIFICATION OF AWARD

25.1 	Prior to the expiration of the period of bid validity, the Company will notify the successful bidder in writing by registered letter or by mail, to be confirmed, that its bid has been accepted.

25.2	The notification of award will constitute the formation of the contract

SECTION III

GENERAL CONDITIONS OF CONTRACT

1. DEFINITIONS

1.1	In this contract the following terms shall be interpreted as indicated:

(a)  “The Contract” means the agreement entered into between the Company and the Contractor as recorded in the Contract Form signed by the parties, including all the attachments and appendices thereto and all documents incorporated by reference therein;

(b)  “The Contract Price” means the price payable to the Contractor under the Contract for the full and proper performance of its contractual obligations;

(c)  “The Materials” means all the products and/or other materials which the Contractor is required to be used in the work under the Contract;

(d)   “Services” means services ancillary to the works,  other incidental services, such as installation, commissioning, provision of technical assistance, and other obligations of the Contractor covered under the contract.

(e)   “GCC” means the General Conditions of Contract contained in this section.

(f)    “SCC” means the Special Conditions of Contract.

(g)   “The Company” means the Organization contracting the work, as named in SCC;

(h)   “The Contractor” means the individual or firm undertaking the work / Service under this Contract;

(i)  “Day” means calendar day.

(j)    “Handing over period” means the period applicable upto completion of work and testing by the contractor at the required location mentioned in service purchase order and accepted by the Company.

(k)   “Earnest Money Deposit” (EMD) means Bid Security / monetary or financial guarantee  to be  furnished by a  tenderer along with its tender”.

2.  APPLICATION

2.1	These General Conditions shall apply to the extent that they are not superseded by provisions in other parts of the Contract.

3. STANDARDS

3.1	The Work executed under this Contract shall conform to the standard requirements mentioned in the schedule.

4. USE OF CONTRACT DOCUMENTS AND INFORMATION

4.1 	The Contractor shall not, without the Company’s prior written consent, disclose the Contract, or any provision thereof, or any specification, plan, drawing, pattern, sample or information furnished by or on behalf of the Company in connection therewith, to any person other than a person employed by the Contractor in performance of the Contract. Disclosure to any such employed person shall be made in confidence and shall extend only so far as may be necessary for purposes of such performance.

4.2	The Contractor shall not, without the Company’s prior written consent, make use of any document or information enumerated in GCC Clause 4.1 except for purposes of performing the Contract.

4.3	Any document, other than the Contract itself, enumerated in GCC clause 4.1 shall remain the property of the Company and shall be returned (in all copies) to the Company on completion of the contractor’s performance under the Contract if so required by the Company.

5. PATENT RIGHTS

5.1	Deleted

6. INSPECTION AND TESTS

6.1	The Company or its representative shall have the right to inspect and/or to test the materials to confirm their conformity to the Contract at no extra cost of the Company. The Special conditions of Contract and/or the Technical Specifications shall specify what inspections and tests the Company requires and where they are to be conducted. The Company shall notify the Contractor in writing of the identity of any representatives retained for these purposes.

6.2	The inspections and test may be conducted on the premises, at point of completion and/or at the time of handing over the execution. The Contractor or its subcontractor(s), all reasonable facilities and assistance shall be extended to the inspectors at no charge to the Company.

6.3	Should any inspected or tested materials/work fail to meet with the schedule requirement, the Company may reject them and the Contractor shall either replace the rejected materials or make all alternations necessary to meet schedule requirements free of cost to the Company.

  - The Company’s right to inspect, test and, where necessary, reject the materials/work at site shall in no way be limited or waived by reason of the Materials having previously been inspected, tested and passed by the Company or its representative.

7.   PACKING

7.1 	Deleted

8   DELIVERY AND DOCUMENTS

8.1	Deleted

9.  INCIDENTAL SERVICES

9.1 	The contractor may be required to provide any or all of the services with respect to this tender, including additional services, if any, specified in SCC:

10. WARRANTY

10.1 	The Contractor warrants that the Materials used under this Contract are new, unused, of the most recent or current models and incorporate all recent improvements in design and materials unless provided otherwise in the Contract. The contractor further warrants that the Materials supplied under this Contract shall have no defect arising from design, materials or workmanship (except when the design and/or material is required by the Company’s specifications) or from any act or omission of the Contractor, that may develop under normal use of the supplied Materials in conditions obtaining in the state/country of final destination.

10.2 	This warranty shall remain valid for one year after work completion handing over and accepted as indicated in the Contract.

10.3 	The Company shall promptly notify the contractor in writing of any claims arising under this warranty.

10.4 	Upon receipt of such notice, the Contractor shall, within the period specified in SCC and with all reasonable speed, repair or replace the defective Materials or parts thereof, without cost to the Company

10.5 	If the Contractor, having been notified, fails to remedy the defect(s) within the period specified in SCC within a reasonable period, the Company may proceed to take such remedial action as may be necessary, at the Contractor’s risk and expense and without prejudice to any other rights which the Company may have against the Contractor under the contract.

11. PAYMENT

11.1	The method and conditions of payment to be made to the Contractor under the Contract shall be specified in the Special Conditions of Contract.

11.2   Payment shall be made in Indian Rupees

12. PRICES

12.1	Prices charged by the Contractor for  Work/Services performed under the Contract shall not vary from the prices quoted/agreed by the Contractor in its bid, with the exception of any prices adjustments authorized in the special Conditions of Contract or in the Company’s request for bid validity extensions, as the case may be.

13. CONTRACT AMENDMENTS

13.1 	Subject to GCC Clause 17, no variation in or modification of the terms of the Contract shall be made except by written amendment signed by the parties.

14. ASSIGNMENT

14.1 	The Contractor shall not assign, in whole or in part, its obligations to perform under the contract, except with the Company’s prior written consent.

15. SUBCONTRACTS

15.1	The supplier shall notify the Company in writing of all subcontracts awarded under the contract if not already specified in his bid. Such notification, in his original bid or later, shall not relieve the Supplier from any liability or obligation under the contract.

16. LIQUIDATED DAMAGES

16.1	If the Contractor fails to perform the contract/services within the period(s) specified in the Contract, the Company shall, without prejudice to its other remedies under the Contract, deduct from the Contract Price, as liquidated damages, a sum equivalent to the percentage specified in SCC of the delivered price of the delayed unperformed contract/Services for each week or part thereof of delay until actual handing over or performance, up to a maximum deduction of the percentage specified in the SCC if any. Once the maximum is reached, the Company may consider termination of the Contract.

17. TERMINATION BY DEFAULT

17.1	The Company may, without prejudice to any other remedy for breach of contract, by written notice of default sent to the contractor, terminate the Contract in whole or part;

(a) if the Contractor fails to complete any or all of the work within the time period(s) specified in the Contract, or within any extension thereof granted by the Company, or

(b) if the Contractor fails to perform any other obligation(s) under the contract.

17.2	In the event the Company terminates the Contract in whole or in part,  the Company may execute, upon such terms and in such manner as it deems appropriate, Work or Services similar to those unperformed, and the Contractor shall be liable to the Company for any excess costs for such similar Works. However, the Contractor shall continue the performance of the Contract to the extent not terminated.

18.  FORCE MAJEURE

18.1	For purposes of this Clause “Force Majeure” means an event beyond the control of the Contractor and not involving the Contractor’s fault or negligence and not foreseeable. Such events may include, but are not limited to, acts of the Company either in its sovereign or contractual capacity, wars or revolutions, fires, floods, epidemics, quarantine restrictions and freight embargoes.

18.2	If a Force Majeure situation arises, the Contractor shall promptly notify the Company in writing of such conditions and the cause thereof. Unless otherwise directed by the Company in writing, the Contractor shall continue to perform its obligations under the Contract as far as is reasonably practical, and shall seek all reasonable alternative means for performance not prevented by the force majeure event.

19. TERMINATION FOR INSOLVENCY

19.1 	The Company may at any time terminate the Contract by giving written notice to the Contractor, if the Contractor becomes bankrupt or otherwise insolvent. In this event, termination will be without compensation to the Contractor, provided that such termination will not prejudice or affect any right of action or remedy, which has accrued or will accrue thereafter to the Company.

20. RESOLUTION OF DISPUTES/ ARBITRATION

20.1 	The Company and the contractor shall make every effort to resolve amicably by direct informal negotiation any disagreement or dispute arising between them under or in connection with the Contract.

  - If, after thirty (30) days from the commencement of such informal negotiations, the Company and the Contractor have been unable to resolve amicably a Contract dispute, either party may require that the dispute be referred for resolution to the formal mechanisms specified in the General Conditions of Contract (Arbitration Clause 20.3). These mechanisms may include, but or not limited to, conciliation mediated by a third Party, adjudication in an agreed national forum, and national arbitration.

20.3 If,  HLL and the Contractor have been unable to resolve amicably a Contract dispute, such dispute or disagreement shall be settled in accordance with the Arbitration and Conciliation Act 1996 the Arbitral Tribunal shall consist of 3 Arbitrator , one each to be nominated by the Company and the contractor. The third Arbitrator shall be chosen by the two Arbitrators so appointed by the parties and shall act as Presiding Arbitrator. In case of failure of the two Arbitrator appointed by the parties to reach upon a consensus within a period of 30 days from the appointment of the Arbitrator appointed subsequently, the Presiding Arbitrator shall be appointed by the President of Institute of Engineers (India).

20.4 The jurisdiction of any disputes, suits and proceeding arising out of the tender shall be only in the court of Thiruvananthapuram.

21. GOVERNING LANGUAGE

21.1 	The contract shall be written in English language. English language version of the Contract shall govern its interpretation. All correspondence and documents pertaining to the Contract which are exchanged by the parties shall be written in the same language.

22. APPLICABLE LAW

22.1 	The Contract shall be interpreted in accordance with the laws of the Union of India.

23. NOTICES

23.1 	Any notice given by one party to the other pursuant to this Contract shall be sent to other party in writing or by cable and confirmed in writing to the other Party’s address specified in Special Conditions of Contract.

23.2 	A notice shall be effective when delivered or on the notice’s effective date, whichever is later.

24. TAXES AND DUTIES

24.1	Contractors shall be entirely responsible for all taxes, licenses, statutory levies etc., incurred until handing over of the contracted site / service to the Company.

SECTION IV

SPECIAL CONDITIONS OF CONTRACT

The following Special Conditions of Contract shall supplement the General Conditions of Contract. Whenever there is a conflict, the provisions herein shall prevail over those in the General Conditions of contract. The Corresponding clause number of the General Conditions is indicated in parentheses.

- The work shall be completed within 20 Days from the date of receipt of firm Work Order / L.O.I.

- The contractor may inspect the site  at Kerala before quoting the work

- During the execution of the work the contractor or authorized representative should be present at site.

- Statutory deduction like Income tax, Work Tax, Service Tax, ESI, PF etc will be deducted from the Contractor’s bill as per the statutory rules.

- Security deposit: 5% of executed value shall be deducted from the first bill and shall be released after defect liability period of ONE year.

- Payment shall be released in two parts - 50% of the total value of work may be released upon the completion of 60% of the work and the final payment (balance 50%) shall be made after successful completion of the total work. The second and final payment shall be released only after clearing all the debris/balance materials from the site. The bills are to be certified by the following official from HLL.

- Electrical Engineer, CMO, Chennai

- Project Engineer (Civil), CMO, Chennai.

Countersigned by any one of the following Official from HLL

- Business Manager (Projects & Services), CMO, Chennai

- Deputy General Manager (MKTG), CMO, Chennai

- All the materials and tools required for the work shall be procured by the contractor and brought to the site to complete the work in the time frame. Entry of men/ materials should be done through the security gate only and the details of the same shall be recorded in the register kept at the security office and the copy of details shall be submitted to the Projects in-charge. All the material brought to the work shall be as per the schedule and samples of materials shall be handed to the project in-charge.

- The contractor has to arrange all the necessary insurance coverage for the materials/ machines / work men deployed by him.

- Measurement is to be recorded in each stage of the work and the payment will be made on actual measurement of the work as per the order.

.

- If the Contractor fails to execute any or all of the work or perform the Services within the time period(s) specified in the Tender/Purchase Order / Work order, the HLL shall, without prejudice to its other remedies under the Contract, deduct from the Contract Price, as penalty, a sum equivalent to the percentage specified in the various terms and conditions of the contract.

- A delay by the Contractor in the performance of its contract obligations shall render the Contractor liable to the imposition of penalty pursuant to this tender/Work Order/contract, unless an extension of time is agreed upon pursuant to agreement/ Work Order without the application of liquidated damages.

- Penalty shall be imposed as per the company rules for any delay committed by the contractor in completing the work as per the order. (Ref. Clause.16 - LIQUIDATED DAMAGES)

- HLL reserves the right to accept or reject the tender either partially or fully without assigning any reason what so ever.

- CONTRACT AMENDMENTS

No variation in or modification of the terms of the Contract shall be made except by written amendment signed by the parties.

15. ASSIGNMENT

The Contractor shall not assign, in whole or in part, its obligations to perform under the contract, except with the HLL’s prior written consent.

16. LIQUIDATED DAMAGES

If the Contractor fails to perform the Services within the period(s) specified in the Contract, the HLL shall, without prejudice to its other remedies under the Contract, deduct from the Contract Price, as liquidated damages, a sum equivalent to 0.5 percentage of the value of the delayed Work or unperformed Services for each week or part thereof of delay until actual completion of performance, up to a maximum deduction of 10 percentage. Once the maximum is reached, the HLL may consider termination of the Contract.

.

SECTION V

QUALIFICATION CRITERIA

(Referred to in Clause 1 of ITB Eligible Bidders)

- For Civil

- The bidder should have Minimum 3 years experience in the relevant field.

- The Tenderer should have executed satisfactorily at least 2 jobs preferably of similar kind in the last 3 years, each contract valued atleast Rs. 14 Lakhs.

- For Electrical

- The bidder should have Minimum 3 years experience in the relevant field.

- The Tenderer should have executed satisfactorily at least 2 jobs preferably of similar kind in the last 3 years, each contract valued atleast Rs. 4 Lakhs

- The tenderer should have authorized person having valid “B” class license or above. The documentary proof with details of such person along with license holder’s consent letter shall be submitted in the technical bid.

- The tenderer should have one Electrical Wireman holding valid Electrical Wireman Permit. The documentary proof with details of the person shall be submitted in the technical bid.

The bidder should fulfil all the above mentioned criteria and proof of such document shall be submitted long with the bid, failing which bid will be treated as non-responsive.

SECTION VI

PROFORMA ABOUT THE CONTRACTOR

HLL/AFT-CMO/KL-DU/CIV&ELE/2017-18/01, DT. 01.04.2017

(Please attach detailed proforma incorporating the information given below)

Tender No……………………….

DATE OF OPENING……..

NAME OF THE TENDERER:……………………………………..

- Name & Full address of the Contractor:

(a)  PAN No

(b)  Service Tax  Regn. No:

(C) TIN No   *****

(d) CST No  *****

(specify whether registered with Kerala State. Attach relevant copies as proof of evidence)

- Any other details

- Specify whether SSI / MSME unit

2.	(a) Telephone No.  				Office/Personal

(b) Fax No.					Office/Personal

(c) e-mail address

- Details of Staff:

- Electrical worker or supervisor (as applicable) to execute this project

(copies of valid license of worker / contractor, issued by statutory authority must be attached along with technical bid)

- Civil worker or supervisor (as applicable) to execute this project

a) technical :

b)  skilled    :

c) unskilled  :

SEAL OF THE  TENDERER

SIGNATURE

NAME AND ADDRESS OF TENDERER

SECTION VII

PROFORMA FOR PERFORMANCE STATEMENT

Proforma for Performance Statement (for a period of last 3 years)

HLL/AFT-CMO/KL-DU/CIV&ELE/2017-18/01, DT. 01.04.2017

Name of the Tenderer:………………………………………………………………..

Name & Address of the Tenderer / Service Provider:……………………………

…………………………………………………………………………………………….

| Order placed by (Full Address of Company / Consignee) | Order No. and Date | Description and quantity of ordered work and service | Value of order | Date of Completion of Contract |  | Remarks indicating reasons for delay, if any | Have the works been satisfactorily maintained? (Attach a documentary proof) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | As per Contract | Actual |  |  |

We hereby certify that if at any time, information furnished by us is proved to be false or incorrect, we are liable for any action as deemed fit by the Company in addition to forfeiture of earnest money.

Signature and Seal of the Bidder ...................................

........................................................................................

SECTION VIII

## HLL LIFECARE LIMITED

Akkulam Factory

THIRUVANANTHAPURAM-695017

HLL/AFT-CMO/KL-DU/CIV&ELE/2017-18/01, DT. 01.04.2017

INDEMINITY CLAUSE

If the contractor fails to execute the order within the time prescribed for the handing over of works ordered or violates or infringes the existing rates as agreed to as mentioned in the work order, the contractor shall and will indemnify the company against all loses or damages whatsoever to be incurred or sustained including the legal cost or expenses incurred by the company by reason of non-handing over of works at agreed quantity and rate with in the time specified in the work order.  The company will initiate legal action if the contractor fails to execute the work order as per the schedule in the work order for the actual loss suffered. No quantity tolerance will be permitted in this regard unless otherwise prior approval is taken by the company before executing any excess quantity and shall be at the risk of the contractor.   Responsiveness of the Bid shall be at the discretion of HLL.

The contractor shall have no right to change the quantity stipulated in the work order.

Bid pronounced Non Responsive by HLL shall be summarily rejected.

The decision of HLL will be final and no correspondence of this shall be entertained.

We have read and understood the above conditions and agree to abide   by the same.

PLACE:                                           NAME AND SIGNATURE OF THE TENDERER

DATE:                                                                                (WITH OFFICE SEAL)

## SECTION IX

## HLL LIFECARE LIMITED

Akkulam Factory

THIRUVANANTHAPURAM-695017

HLL/AFT-CMO/KL-DU/CIV&ELE/2017-18/01, DT. 01.04.2017

#### DECLARATION

I / WE, The Bidder undertake, we shall execute, within the purview of the contract, all the works and activities includes; transportation, loading, unloading and other technical work for the installation of the equipment at the designated site as per the schedule/contract /work order, at Kerala.

We confirm having read and understood all the schedule requirements, instructions, forms, terms and conditions and other requirements of the above tender (both expressed and implied) in full and that we agree to abide by all without any deviation.

SEAL OF THE TENDERER

SIGNATURE

NAME AND ADDRESS OF TENDERER

PART : B

SECTION  VIII		ABSTRACT PRICE SCHEDULE

SECTION IX			PRICE SCHEDULE

SECTION  X

- HLL LIFECARE LIMITED

AKKULAM FACTORY,  THIRUVANANTHAPURAM-695017

RENOVATION OF DIALYSIS ROOM AT PRIMARY HEALTH CENTRE, EDAKKADU, KANNUR DISTRICT, KERALA  ON TURNKEY BASIS

- (CIVIL AND ELECTRICAL WORKS)

- ABSTRACT PRICE BID

| Site. No. | Details of Hospitals | Amount Quoted (Rs) |
| --- | --- | --- |
| 1 (a) | Civil Work |  |
| 1 (b) | Electrical Work |  |
| Total Rs: |  |  |

Total words in Rupees:___________________________________________________________

Bidder has to quote as per the Make/Brand specified in the BOQ, proposed to use for the project, and has to marks the Make/Brand in the quote / price schedule.

SEAL OF THE TENDERER

Date:                                                                                                                           SIGNATURE

NAME AND ADDRESS OF TENDERER

SECTION  XI

- HLL LIFECARE LIMITED

AKKULAM FACTORY, THIRUVANANTHAPURAM-695017

Ph: 0471 244 5930 /244 2642

hcdcmo@lifecarehll.com

RENOVATION OF DIALYSIS ROOM AT PRIMARY HEALTH CENTRE, EDAKKADU, KANNUR DISTRICT, KERALA  ON TURNKEY BASIS

- (CIVIL WORKS)

PRICE SCHEDULE (1a)

| No | Description of work | Qty | Unit | Unit Rate in Rs. | Total Rate in Rs. |
| --- | --- | --- | --- | --- | --- |
| 1 | Cleaning the site and removing debris with carriege of meterial by manual labour including loading ,unloading and stacking for lead lass than 0.5 KM,also cleaning the renovated area including cleaning of wall t,floor tiles ,plumbing meterials,windows ,dooors etc complete whichever in that area new/exsisiting after the completion of works as per the direction of site in charge |  | Ls |  |  |
| 2 | Dismantling doors, windows and clerestory windows (steel aluminum or wood) shutter including chowkhats, architrave, holdfasts etc. complete and stacking within 50 metres lead Of area 3 sq. metres and below | 11.00 | Each |  |  |
| 3 | Dismantling tile work in floors and roofs laid in cement mortar includingstacking material within 50 metres lead. | 4.00 | Sqm |  |  |
| 4 | Demolishing brick work manually/ by mechanical means including stackingof serviceable material and disposal of unserviceable material within 50 metres lead as per direction of Engineer-in-charge.In cement mortar | 8.00 | cum |  |  |
| 5 | Demolishing R.C.C. work by mechanical means and stockpiling atdesignated locations and disposal of dismantled materials up to a lead of1 kil ometre, stacking serviceable and unserviceable material separatelyincluding cutting reinforcement bars. | 1.00 | cum |  |  |
| 6 | Earth work in excavation by manual means (exceeding 30cm in depth. 1.5m in width as well as 10 sqm on plan) including disposal of excavated earth, lead upto 50m and lift upto 1.5m, disposed earth to be levelled and neatly dressed:All kinds of soil | 5.00 | cum |  |  |
| 7 | Filling available excavated earth (excluding rock) in trenches, plinth, sides of foundations yard levelling etc., in layers not exceeding 20cm in depth: consolidating each deposited layer by ramming and watering etc complete, and form uniform level . | 13.00 | cum |  |  |
| 8 | Providing and laying in position cement concrete of specified grade excluding the cost of centering and shuttering - All work up to plinth level :1:4:8 (1 Cement : 4 coarse sand : 8 graded stone aggregate 40 mm nominal size) | 3.00 | cum |  |  |
| 9 | Random rubble masonry with hard stone in foundation and plinth includinglevelling up with cement concrete 1:6:12 (1 cement : 6 coarse sand : 12graded stone aggregate 20 mm nominal size) upto plinth level with :Cement mortar 1:6 (1 cement : 6 coarse sand) | 7.00 | Sqm |  |  |
| 10 | Brick work with common burnt clay F.P.S. (non modular) bricks of class designation 7.5 in superstructure above plinth level up to floor V level in all shapes and sizes in : |  |  |  |  |
|  | Cement mortar 1:4( 1 cement : 4 coarse sand | 9.00 | Cu.M |  |  |
| 11 | Half brick masonry with common burnt clay F.P.S. (non modular) bricksof class designation 7.5 in superstructure above plinth level up to floor V level. |  |  |  |  |
|  | Cement mortar 1:4( 1 cement : 3 coarse sand | 4.00 | Sqm |  |  |
| 12 | 12mm cement plaster on the rough side of single or half brick wall of mix : 1 : 4 (1 cement : 4 fine sand) for internal walls, column and beams etc | 97 | Sqm |  |  |
| 13 | Reinforced cement concrete work in walls (any thickness), including attached pilasters, buttresses, plinth and string courses, fillets, columns,pillars, piers, abutments, posts and struts etc. up to floor five level,excluding cost of centering, shuttering, finishing and reinforcement : |  |  |  |  |
|  | 1:1.5:3 (1 cement : 1.5 coarse sand : 3 graded stone aggregate20 mm nominal size) | 1.00 | cum |  |  |
| 14 | Centering and shuttering including strutting, propping etc. and removal of form for : |  |  |  |  |
|  | Suspended floors, roofs, landings, balconies and access platform | 9.00 | sqm |  |  |
| 15 | Steel reinforcement for R.C.C. work including straightening, cutting,bending, placing in position and binding all complete upto plinth level & above plinth level. |  |  |  |  |
|  | hermo-Mechanically Treated bars | 99.00 | Kg |  |  |
| 16 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) : |  |  |  |  |
| a). | For Fixed Portion , Powder coated aluminium (minimum thickness of powder coating 50 micron) | 329.00 | Kg |  |  |
| b). | For shutters of doors, windows & ventilators including providing and fixing hinges/ pivots and making provision for fixing of fittings wherever required including the cost of EPDM rubber / neoprene gasket required (Fittings shall be paid for separately). Powder coated aluminium (minimum thickness of powder coating 50 micron) | 309.00 | Kg |  |  |
| 17 | Providing and fixing 3 mm Aluminum composite pannel, of approved brand and manufacture in panelling fixed in aluminium doors,windows shutters and partition frames with C.P.brass/stainless steel screws etc. complete as per architectural drawings and directions of engineer-in-charge. |  |  |  |  |
|  | Prelaminated particle board with decorative lamination on both sides | 22.00 | sqm |  |  |
| 18 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc with PVC/neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge. |  |  |  |  |
| a) | With float glass panes of 5.5 mm thickness | 44.00 | sqm |  |  |
| 19 | Providing and fixing aluminium tower bolts ISI marked anodised ( anodic coating not less than grade AC 10 as per IS: 1868) transparent or dyed to required colour or shade with nuts and screws etc. complete- |  |  |  |  |
| a) | 250 X 10 mm | 12 | each |  |  |
| b). | 150x10mm | 12 | each |  |  |
| 20 | Providing and fixing 100mm brass locks (best make of approved quality) for aluminium doors including necessary cutting and making good etc. complete. | 5 | each |  |  |
| 21 | Providing and fixing aluminium handles ISI marked anodised ( anodic coating not less than grade AC 10 as per IS: 1868) transparent or dyed to required colour or shade with nuts and screws etc. complete- |  |  |  |  |
| a) | 125 mm | 24 | each |  |  |
| 22 | Providing and fixing 18mm thick gang saw cut mirror polished premoulded and prepolished) machine cut for kitchen platforms, vanity counters, window sills , facias and similar locations of required size of approved shade, colour and texture laid over 20mm thick base cement mortar 1:4 (1 cement : 4 coarse sand) with joints treated with white cement, mixed with matching pigment, epoxy touch ups, including rubbing, curing, moulding and polishing to edges to give high gloss finish etc. complete at all levels. Granite of any colour and shade: Area of slab over 0.50 sqm.: Lowest retail price not less than Rs. 2000.00/m2( Granite to be selected by the Engineer in charge) | 3.00 | sqm |  |  |
| 23 | Providing edge moulding to 18 mm thick marble/granite stone counters, Vanities etc., including machine polishing to edge to give high gloss finish etc. complete as per design approved by Engineer-in-Charge. | 8.00 | metre |  |  |
| 24 | Providing and laying joint free Vitrified anti skid matt finished tiles in different sizes (thickness to be specified by manufacturer) with water absorption less than 0.08 % and conforming to I.S. 15622, of approved make (Johnson & Johnson/Kajariea/NItco) in all white/off white colours & shade in skirting, riser of steps, over 12 mm thick bed of cement mortar 1:3 (1 cement: 3 coarse sand), including grouting the joint with white cement & matching pigments etc. complete.Size of Tile 600x600 mm (Lowest retail price not less than Rs. 880/m2) (Tiles to be selected by the Engineer in charge) for Labor room | 143.00 | Sqm |  |  |
| 25 | Providing and fixing tiled false ceiling of approved materials of size 595x595 mm in true horizontal level suspended on inter locking metal grid of hot dipped galvanized steel sections ( galvanized @ 120 gsm/sqm, both side inclusive) consisting of main "T" runner with suitably spaced joints to get required length and of size 24x38mm made from 0.30mm thick (minimum) sheet, spaced at 1200mm center to center and cross "T" of size 24x25mm made of 0.30mm thick (minimum) sheet, 1200mm long spaced between main "T" at 600mm center to center to form a grid of 1200x600 mm and secondary cross "T" of length 600mm and size 24x25mm made of 0.30 mm thick (minimum) sheet to be interlocked at middle of the 1200x600mm panel to form grids of 600x600mm and wall angle of size 24x24x0.3 mm and laying false ceiling tiles of approved texture in the grid including, wherever, required, cutting/making, opening for services like diffusers, grills, light fittings, fixtures, smoke detectors etc. |  |  |  |  |
|  | Main "T" runners to be suspended from ceiling using GI slotted cleats of size 27 x 37 x 25 x1.6 x mm fixed to ceiling with 12.5 mm dia and 50 mm long dash fasteners, 4mm GI adjustable rods with galvanised butterfly level clips of size 85 x 30 x 0.8 mm spaced at 1200mm center to center along main T, bottom exposed width of 24 mm of all T-sections shall be pre-painted with polyester paint, all complete at all heights as per specifications drawings and as directed by Engineer-in-charge. |  |  |  |  |
|  | 12.5 mm thick square edge PVC Laminated Gypsum Tile of size 595x595 mm, made of Gypsum plasterboard,manufactured from natural gypsum as per IS 2095 part I and laminated with white 0.16mm thick fire retardant PVC film on the face side and 12micron metalized polyester on the back side with all edges sealed with the face side PVC film which goes around and wraps the edges and is bonded to the edges and the back side metalized polyester film so as to make the tile a completely sealed unit. | 81.00 | Sqm |  |  |
| 25 | Providing and laying rectified Anti skid Glazed Ceramic floor tiles 300x300mm or more (thickness to be specified by the manufacturer) of 1 st quality conforming to IS : 15622 of approved make in colours white,Ivory, Grey, Fume, Red Brown, laid on 20mm thick cement mortor 1:4 (1 cement: 4 coarse sand)including grouting the joints with white cement and matching pigments etc.complete. | 23.00 | sqm |  |  |
| 26 | Providing and fixing Ist quality ceramic glazed wall tiles conforming to IS:15622 (thickness to be specified by the manufacturer), of approved make,in all colours, shades except burgundy, bottle green, black of any size as approved by Engineer-in-Charge, in skirting, risers of steps and dados,over 12 mm thick bed of cement mortar 1:3 (1 cement : 3 coarse sand) and jointing with grey cement slurry @ 3.3kg per sqm, including pointingin white cement mixed with pigment of matching shade complete. | 289.00 | sqm |  |  |
| 27 | Providing and fixing M.S. grills of required pattern in frames of windows Fixed to openings /wooden frames with rawl plugs screws etc with M.S. flats, square or round bars etc. including priming coat withapproved steel primer all complete. | 78.00 | kg |  |  |
| 28 | Removing white or colour wash by scrapping and sand papering and preparing the surface smooth including necessary repairs to scratches etc. complete | 526.00 | sqm |  |  |
| 29 | Applying one coat of water thinnable cement primer of approvedbrand and manufacture on wall surface ,Water thinnable cement primer | 566.00 | sqm |  |  |
| 30 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 526.00 | sqm |  |  |
| 30 | Wall painting with acrylic emulsion paint of approved brand and manufacture to give an even shade –Two or more coats on new work.-internal walls | 526.00 | sqm |  |  |
| 31 | Finishing walls with Acrylic Smooth exterior paint of required shade : | 90.00 | sqm |  |  |
| 32 | Painting wood work with Deluxe Multi Surface Paint of required shade. Two or more coat applied @ 0.90 ltr/10 sqm over anunder coat of primer applied @0.75 ltr/ 10 sqm of approved brand and manufacture | 31.00 | sqm |  |  |
| 33 | Painting Steel work with Delux Multi surface paint to give an even shade.Two or more coat applied @ 0.90 ltr/10sqm over an over an under coat of primer applied @ 0.80ltr/10aqm of approved brand or manufacture. | 62.00 | sqm |  |  |
| 34 | Providing and fixing factory made PVC door frame made of PVC extruded section having an overall dimensions as below ( tolerance ± 1mm ), with wall thickness 2.0mm (± 0.2mm), corners of the door frame to be joined with galvanized brackets and stainless steel screws, joints mitered and plastic welded. The hinged side vertical of the frames reinforced by galvanized M.S. tube of size 19x19 mm and 1mm (± 0.1mm) wall thickness and 3 nos. Stainless steel hinged fixed to the frame complete as per manufacturers Specification and direction of engineering -in-change : Extruded section profile size 48x40 mm | 16.00 | m |  |  |
| 35 | Providing and fixing existing door frames: 24 mm thick factory made PVC door shutters made of styles and rails of a UPVC hollow section of size 59x24 mm and wall thickness 2 mm (± 0.2mm) with in built edging on both sides. The styles and rails mitered and joint at the corners by means of M.S. galvanized / plastic brackets of size 75x220 mm having wall thickness 1.0 mm and stainless steel screws. The styles of the shutter reinforced by inserting galvanized M.S. tube of size 20x20 mm and 1mm (± 0.1mm) wall thickness. The lock rail made up of " H" section, a UPVC hollow section of size 100x24 mm and 2mm (± 0.2mm) wall thickness, fixed to the shutters styles by means of plastic/ galvanized M.S.'U' cleats. The shutter frame filled with a UPVC multi chambered single panel of size not less than 620mm, having over all thickness of 20mm and 1mm (± 0.1mm) wall thickness. The panes filled vertically and tie bar at two places by inserting horizontally 6mm galvanized M.S. rod and fastened with nuts and washers, complete as per manufacturer's specification and direction of Engineer- in - charge. (for W,C and bath room door shutter). | 6.00 | m2 |  |  |
| 36 | Structural steel work riveted, bolted or welded in built up sections, trusses and framed work, including cutting, hoisting, fixing in position and applying a priming coat of approved steel primer all complete. | 434.00 | Kg |  |  |
|  | Providing corrugated Aluminium sheet (0.50mm thk) roofing including vertical / curved surfacefixed with polymer coated J or L hooks, bolts and nuts 8 mm diameterwith bitumen and G.I. limpet washers or with G.I. limpet washers filledwith white lead, including a coat of approved steel primer and two coatsof approved paint on overlapping of sheets complete (up to any pitch inhorizontal/ vertical or curved surfaces), excluding the cost of purlins, raftersand trusses and including cutting to size and shape wherever required. |  |  |  |  |
| 37 | 1.00 mm thick with zinc coating not less than 275 gm/m² | 29.00 | Sqm |  |  |
| 38 | Supplying and providing auto closer for aluminium doorsFully controlled closing with adjustable speed between 180° - 20° , good quality meterial like brand- Dorma or equelent , Capacity of meterila should be according to door size, and the directions of Engineer-in-charge | 5 | each |  |  |
| 39 | Suppying and providing elbow action surgical tap confirming with ISI branded like jaquar ,grohe etc | 2 | each |  |  |
| 40 | Supplying and providing curtains for doors and windows using good quality type standered meterilas ,colour and specification of meterilas should be as per the instruction of site in charge | 5.00 | sqm |  |  |
|  | Plumbing works |  |  |  |  |
| 40 | Providing and fixing gun metal gate valve with C.I. wheel of approved quality -25MM | 3 | sqm |  |  |
| 41 | Providing and fixing C.P. brass angle valve for basin mixer and geyserpoints of approved quality conforming to IS:8931 a) 15 mm nominal bore: 15 mm nominal bore.Make:Hindware,Cera,Jaquar etc.. | 6 | Nos. |  |  |
| 42 | Providing and fixing PTMT Bottle Trap for Wash basin and sink.Bottle trap 38mm single piece moulded with height of 270mm, effective length of tail pipe 260mm from the centre of the waste coupling 77mm breadth with 25mm minimum water seal, weighing not less than 263gms. | 2 | Nos |  |  |
| 43 | Providing and fixing trap of self cleansing design with screwed down orhinged grating with or without vent arm complete, including cost of cuttingand making good the walls and floors :100 mm inlet and 100 mm outlet | 3 | Nos |  |  |
| 44 | Constructing brick masonry manhole in cement mortar 1:4 ( 1 cement : 4 coarse sand ) with R.C.C. top slab with 1:2:4 mix (1 cement : 2 coarse sand : 4 graded stone aggregate 20 mm nominal size), foundation concrete 1:4:8 mix (1 cement : 4 coarse sand : 8 graded stone aggregate 40 mm nominal size), inside plastering 12 mm thick with cement mortar 1:3 (1 cement : 3 coarse sand) finished with floating coat of neat cement and making channels in cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20 mm nominal size) finished with a floating coat of neat cement complete as per standard design : With common burnt clay F.P.S designation 7.5 | 1 | Nos |  |  |
| 45 | Providing and fixing square – mouth PVC. Gully trap grade “A” complete with CI grating brick masonry chamber with water tight tight CI cover with frame of 300 x 300mm size (inside) the weight of cover to be not less than 4.50 kg and frame to be not less than 2.70 kg as per standard design | 2 | Nos |  |  |
| 46 | Providing and fixing white vitreous china pedestal type water closet(European type W.C. pan) with seat and lid, 10 litre low level white P.V.C.flushing cistern, including flush pipe, with manually controlled device(handle lever), conforming to IS : 7231, with all fittings and fixturescomplete, including cutting and making good the walls and floors wherever required :W.C. pan with ISI marked white solid plastic seat and lid | 3 | Nos |  |  |
| 47 | Providing and fixing wash basin with C.I. brackets, 15 mm C.P. brasspillar taps, 32 mm C.P. brass waste of standard pattern, including paintingof fittings and brackets, cutting and making good the walls whereverrequire:White Vitreous China Wash basin size 630x450 mm with apair of 15 mm C.P. brass pillar taps | 2 | Nos |  |  |
| 48 | Providing and fixing C.P. brass long body bib cock of approved quality conforming to IS standards and weighing not less than 690 gms. 15 mm nominal bore | 3 | Nos. |  |  |
| 49 | Providing & fixing of CP Health faucet. Hand operated spray unit (instand type) | 1 | Set |  |  |
| 50 | Providing and fixing PVC pipes for water supply arrangements including cost pipes, fittings, solvent cement,testing of pipes, labour charges etc complete : Concealed work including cutting chases and making good the wall etc |  |  |  |  |
| a) | 20mm dia. PVC pipe | 15 | Mtr. |  |  |
| b) | 25mm dia. PVC pipe | 20 | Mtr. |  |  |
| c) | 32mm dia. PVC pipe | 15 | Mtr. |  |  |
| 51 | Providing and fixing CP grating of 6mm thick with rim including setting in floor with white cement mortar and making good the floor |  |  |  |  |
| a) | 125mm dia | 4 | Nos. |  |  |
| 52 | Providing and fixing PVC soil, waste and vent pipe to IS : 13592 of SWR quality including all fittings, like : Bends, Tees, Elbows, Collars, junction, inspection doors, cowls, offsets, access pipes, jointing with rubber ring / solvent cement by application of leak proof adhesive like FRP paste including cutting holes in walls and floors and making good. Make:Supreme,Finolex,Prince etc.., |  |  |  |  |
| a) | 50mm dia | 25 | Mtr. |  |  |
| b) | 110mm dia | 25 | Mtr. |  |  |
| 53 | Providing and fixing Chlorinated Polyvinyl Chloride (CPVC) pipes, having thermal stability for hot & cold water supply including all CPVC plain & brass threaded fittings i/c fixing the pipe with clamps at 1.00 m spacing. This includes jointing of pipes & fittings with one step CPVC solvent cement and the cost of cutting chases and making good the same including testing of joints complete as per direction of Engineer in Charge.Concealed work including cutting chases and making good the walls etc |  |  |  |  |
| a) | 20 mm nominal outer dia .Pipes | 20 | metre |  |  |
| b) | 32 mm nominal outer dia Pipes | 15 | metre |  |  |
| C) | 50 mm nominal outer dia Pipes | 15 | metre |  |  |
| 54 | Providing and fixing Stainless Steel A ISI 304 (18/8) kitchen sink as perIS: 13983 with C.I. brackets and stainless steel plug 40 mm, includingpainting of fittings and brackets, cutting and making good the wallswherever required ::(Lowest retail price of sink not less than Rs. 3150) |  |  |  |  |
|  | Kitchen sink without drain board,Size -610x510 mm bowl depth 200 mm | 1 | each |  |  |
| 55 | Providing and fixing PVC Waste pipe for sink or wash basin including PVC waste fittings complete.:Semi rigid pipe 32mm dia | 3 | Nos. |  |  |
| 56 | Providing and fixing CP Waste coupling for wash basin and sink of approved quality, colour and make. Waste coupling 31mm of 79mm length and 62mm breadth, weighing not less than 45 gms. Make:Hindware,Jaquar etc.., | 3 | Nos. |  |  |
| 57 | Extra for providing opening of required size & shape for wash basin/ kitchen sink in kitchen platform, vanity counter and similar location in marble/ Granite/stone work, including necessary holes for pillar taps etc. including moulding, rubbing and polishing of cut edges etc. complete. | 1 | each |  |  |
| 58 | Providing and fixing 600x450 mm beveled edge mirror of superior glass (of approved quality) complete with 6 mm thick hard board ground fixed to wooden cleats with C.P. brass screws and washers complete. | 1 | each |  |  |
| 59 | Providing and fixing PTMT towel rail complete with brackets fixed to wooden cleats with CP brass screws with concealed fitting arrangement of approved quality and colour. 600mm long towel rail with total length of 645mm, width 78mm and effective height of 88mm, weighing not less than 190gms. | 2 | each |  |  |
| 60 | Providing and fixing PTMT liquid soap container 109mm wide, 125mm high and 112mm distance from wall of standard shape with bracket of the same materials with snap fittings of approved quality and colour. weighing not less than 105 gms. | 2 | each |  |  |
|  | Total |  |  |  |  |

SIGNATURE

SEAL OF THE TENDERER				NAME AND ADDRESS OF TENDERER

- SECTION  XI

- HLL LIFECARE LIMITED

AKKULAM FACTORY, THIRUVANANTHAPURAM-695017

Ph: 0471 244 5930 /244 2642

hcdcmo@lifecarehll.com

RENOVATION OF DIALYSIS ROOM AT PRIMARY HEALTH CENTRE, EDAKKADU, KANNUR DISTRICT, KERALA  ON TURNKEY BASIS

- (ELECTRICAL WORKS)

PRICE SCHEDULE (1 B)

| Sl. No | Description of work | Unit | Qty | Unit Rate in Rs. | Total Rate in Rs. |
| --- | --- | --- | --- | --- | --- |
|  | SCHEDULED ITEMS |  |  |  |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 47 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required. |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 205 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 305 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 265 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required. | Point | 3 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Havells) | Each | 15 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand/Havells) |  |  |  |  |
| a | 3 module | Each | 6 |  |  |
| b | 6 module | Each | 33 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand/Havells) |  |  |  |  |
| a | 5/6 Amps switch | Each | 34 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 34 |  |  |
| c | 15/16 Amps switch | Each | 32 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 32 |  |  |
| e | Telephone socket outlet | Each | 2 |  |  |
| 7 | Telephone wiring in existing conduit: |  |  |  |  |
|  | Supplying and drawing following pair 0.5 sq mm FR PVC insulated annealed copper conductor, unarmored telephone cable in the existing surface/ recessed steel/ PVC conduit as required. |  |  |  |  |
| a | 2 Pair | Metre | 50 |  |  |
| 8 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 175 |  |  |
| b | 25 mm | Metre | 125 |  |  |
| 9 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 29 |  |  |
| 10 | ITC Wall fan:Installation, testing and commissioning of ceiling fan, including wiring the down rods of standard length (upto 30 cm) with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable etc. as required. | Each | 10 |  |  |
| 11 | Supplying and fixing stepped type electronic fan regulator on the existing modular plate switch box including connections but excluding modular plate etc. as required. | Each | 0 |  |  |
| 12 | CABLE LAYING |  |  |  |  |
|  | Laying and fixing of one number PVC insulated and PVC sheathed / XLPE power cable of 1.1kV grade of following size clamped on wall surface with 1mm thick saddles as required complete. |  |  |  |  |
| a | From VDB to LDB |  |  |  |  |
|  | 4X10 sq.mm | Metre | 10 |  |  |
| b | From VDB to PDB |  |  |  |  |
|  | 4X16 sq.mm | Metre | 10 |  |  |
| c | From Panel board to UPS AND VDB |  |  |  |  |
|  | 4X16 sq.mm | Metre | 75 |  |  |
| 13 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of PVC insulated and PVC sheathed/XLPE aluminium conductor cable fo 1.1KV grade as required. |  |  |  |  |
| a | 4X10 sq.mm | Each | 2 |  |  |
| b | 4X16 sq.mm | Each | 4 |  |  |
| 14 | CABLE SUPPLY & WIRING FOR UPS |  |  |  |  |
| a | Wiring for circuit/ submain wiring alongwith earth wire with the following sizes of FRLS PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | 4 X 16 sq. mm + 2 X 6 sq. mm earth wire | Metre | 75 |  |  |
|  | Non Scheduled Items |  |  |  |  |
| 15 | Supplying and fixing PVC angle batten/batten holder with 18W CFL(Make:Havells/Philips) including connection as required. | Each | 0 |  |  |
| 16 | Supply and installation of 2x34/36W recess mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required.(Havells2x34 W LED 4000K) ordering code:LHEEBP6CUL1W034)/Bajaj SKYLUX BZRSQL 36W XL NW) .or else any higher versions may be approved as per the direction of engineer in HLL/NHM without change in cost. | Each | 11 |  |  |
| 17 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N) | Each | 13 |  |  |
| 18 | Supply & installation of 1x18 watts recess mounted down light fixture with glass top, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Make: Havells/Bajaj/Philips) | Each | 5 |  |  |
| 19 | Supply and installation of wall fan with its all accessories(Make Havells/bajaj/usha) | Each | 10 |  |  |
| 20 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, Vertical MCB distribution board, 415 volts, on surface/recess, complete with tinned copper busbar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc. as required (with provision for isolator as incomer) (Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 4 way, double door | Each | 1.00 |  |  |
| 20.1 | S/F of FP Isolators |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts isolators in the existing Vertical DB as incomer complete with connections, testing and commissioning etc as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 63 A, 4P isolator | Each | 1.00 |  |  |
| 20.2 | S/F of TP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 63 amps rating, 415 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 32A TP MCB | Each | 1.00 |  |  |
|  | 63A TP MCB | Each | 1.00 |  |  |
| 21 | S/F of TPN MCB PDB |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, MCB distribution board, 415 volts, on surface/recess, complete with tinned copper bus bar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc. as required (But without MCB/ RCBO/isolator) (Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 12 way Double door, horizontal type. | Each | 1.00 |  |  |
| 21.1 | S/F of TPN RCBO |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts RCBO, having a sensitivity current upto 100/300 milliamperes in the existing MCB PDB complete with connections, testing and commissioning etc. as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 63 A 4P RCBO 100mA | Each | 1.00 |  |  |
| 21.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for power loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 36.00 |  |  |
| 22 | TPN MCB LDB |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, MCB distribution board, 415 volts, on surface/recess, complete with tinned copper bus bar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc as required(But without MCB/RCBO /isolator) (Make:Legrand/Havells/ HPL) |  |  |  |  |
|  | 6 way, double door | Each | 1.00 |  |  |
| 22.1 | S/F of TPN RCBO |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts RCBO, having a sensitivity current upto 30 milliamperes in the existing MCB LDB complete with connections, testing and commissioning etc. as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 32 A 4P RCBO 30mA | Each | 1.00 |  |  |
| 22.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 10 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 6/10A SP MCB | Each | 18 |  |  |
| 23 | CABLE SUPPLY |  |  |  |  |
|  | Supplying the following PVC insulated and PVC sheathed/XLPE power cable of 1.1 KV grade as per CPWD standards as required. (Make:Havells/ Glostar/Nicco) |  |  |  |  |
| a | 4X10 sq.mm | Metre | 10 |  |  |
| b | 4X16 sq.mm | Metre | 10 |  |  |
| 24 | Pipe earthing with G.I. earth pipe 2.5 metre, long, 40 mm dia including accessories, and providing masonry enclosure with cover plate having locking arrangement and watering pipe etc. with charcoal/ coke and salt ) as required complete. | Set | 1 |  |  |
| 25 | Supplying and laying 10 SWG Copper wire on, surface or in recess for loop earthing along with existing surface/recessed conduit/submain wiring/ cable as required. | Metre | 80 |  |  |
| 26 | Supply and fixing of powder coated cubicle box with 63 A TPN Switch disconnector fuse and suitable ISI Marked fuses(Make: Legrand/Havells/HPL) | Each | 1 |  |  |
| 27 | WORK COMPLETION |  |  |  |  |
| 28 | Supply and pointing exhaust fan shutter type white colour-9"/10" | Each | 2 |  |  |
| 29 | Supply and Fixing of Calling bell with its all accessories | Each | 1 |  |  |
|  | Total |  |  |  |  |

SIGNATURE

SEAL OF THE TENDERER				NAME AND ADDRESS OF TENDERER

Annexure I

CIVIL DRAWING

Annexure II

ELECTRICAL DRAWING